Portuguese Cushion Triangle

## By Ernie Hawes

With perform instructions

Angles for R.I. = 1.540

94 + 15 girdles = 109 facets

3-fold, mirror-image symmetry

96 index

L/W = 1.028  T/W = 0.608  U/W = 0.589

P/W = 0.489  C/W = 0.161

Vol./W³ = 0.238

PREFORM

pf1	49.20°	16-48-80		Cut to Centerpoint

pf2	48.45°	12-20-44-52-76-84	Cut to centerpoint set by pf1

pf3	46.25°	08-24-40-56-72-88	Cut to established centerpoint

pf4	90.00°	16-48-80		Set stone width; determine girdle

pf5	90.00°	12-20-44-52-76-84	Cut to even girdle line set by pf4

pf6	90.00°	08-24-40-56-72-88	Cut to even girdle line

PAVILION

1	55.00°	16-48-80		Cut enough below pf4 for other preform facets to be eliminated later

2	55.00°	12-20-44-52-76-84	Cut to girdle line

3	55.00°	08-24-40-56-72-88	Cut to girdle line

4	50.00°	10-22-42-54-74-86	Meet 2 & 3 at girdle

5	50.00°	14-18-46-50-78-82	Meet 1 & 2 at girdle and at juncture of 2 & 4

6	49.00°	06-26-38-58-70-90	Meet 3 at girdle

7	47.00°	16-48-80		Meet at juncture of 1 & 5

8	47.00°	12-20-44-52-76-84	Meet at juncture of 2, 4 & 5 and at 4 & 6

9	45.00°	14-18-46-50-78-82	Meet at juncture of 5, 7 & 8

10	43.00°	16-48-80		Meet at juncture of 7 & 9

CROWN

a	43.00°	16-48-80		Determine girdle thickness

b	43.00°	12-20-44-52-76-84	Meet at girdle line established by a

c	43.00°	08-24-40-56-72-88	Meet at girdle line

d	38.00°	10-22-42-54-74-86	Meet a & c at girdle

e	38.00°	14-18-46-50-78-82	Meet a & b at girdle

f	34.00°	16-48-80		Meet a & e

g	33.50°	12-20-44-52-76-84	Meet at juncture of b, d & e

h	29.90°	14-18-46-50-78-82	Meet at juncture of e, f & g

T	00.00°	Table			Cut to juncture of d & g and f & h